DESIGN DOCUMENT

Version 2.0

A Project On

SEISMIC ACQUISITION 3D MAPPING SYSTEM

GROUP – 2 (METAMINDS)

NANDEESWAR BADUGU

SAITEJA SIRIPURAPU

JAYTHRI PUPPALA

SAI KISHORE BHADRAGIRI

Table of Contents

VERSION TABLE2

OVERVIEW2

SYSTEM PURPOSE2

USER INPUTS3

KEY CALCULATIONS3

DATA STRUCTURES4

OUTPUTS4

MATHEMATICAL CONSIDERATIONS4

ALGORITHMS STEPS5

USER INTERFACE5

TESTING AND VALIDATIONS6

DEPENDENCIES6

CONCLUSION6

- VERSION TABLE

| Version | Date | Author | Description of Changes | Reviewed By |
| --- | --- | --- | --- | --- |
| 1.0 | 11/08/2024 | Nandeeswar/ Jaythri | Initial draft of the Design Document. | Mr. Thomas Dillon J |
| 1.1 | 11/18/2024 | Nandeeswar/ Jaythri | Updated input parameters | Mr. Thomas Dillon J |
| 2.0 | 11/26/2024 | Nandeeswar/ Jaythri | Updated user input parameters and removed visualization | Yet to Review |

- OVERVIEW

This document outlines the design and functionality of a software tool that calculates and visualizes the multiplicity and offset distribution in a 3D seismic acquisition system. The tool simulates the relationship between multiple sources and geophones in a seismic survey and calculates two key parameters: multiplicity and offset. These parameters can be printed as a 2D matrix for detailed numerical analysis or visualized directly within the tool for an intuitive representation of the seismic survey layout. The visualization includes color-coded maps highlighting variations in multiplicity and offset distribution across the surveyed area.

- SYSTEM PURPOSE

The tool's purpose is to model and visualize the interaction between seismic sources and geophones in a 3D seismic acquisition system. The primary outputs are:

1. Multiplicity (or Fold):

- Multiplicity refers to the number of times a particular subsurface point is sampled by unique combinations of source and receiver pairs.

- It is essentially a measure of redundancy in the seismic data acquisition process. Higher multiplicity improves the signal-to-noise ratio and helps suppress random noise in the data.

- Mathematically, it is calculated as the count of midpoint traces that overlap a specific subsurface bin or cell in the survey grid.

- Example: In a 3D survey, if a subsurface bin receives seismic reflections from 12 unique source-receiver pairs, the fold or multiplicity of that bin is 12.

2. Offset:

- Offset is the horizontal distance between a seismic source point (SP) and a receiver point (RP) at the surface of the Earth.

- This distance is crucial because it affects the travel time of seismic waves, their reflection angles, and the resolution of deeper subsurface structures.

This tool is particularly useful for seismic survey planning and optimizing the configuration of source and receiver arrays.

- USER INPUTS

- Area: Defines the size of the acquisition area, specifying the spatial extent.

X-Direction (miles/meters): The length of the area in the X direction (inline).

Y-Direction (miles/meters): The length of the area in the Y direction (cross-line).

- Geophone Distance: Specifies the distance between adjacent geophones in meters, determining the density of the grid and how spread out the geophones are.

- Geophone Line Distance: Defines the distance between each line of geophones, helping to determine the coverage and layout of the receiver lines.

- Number of Channels recorded per Line: Indicates the number of channels in each line, which affects the number of receivers that can be recorded simultaneously.

- Number of Lines Recorded at One Time: Specifies how many geophone lines are recorded simultaneously, influencing the number of simultaneous measurements taken.

- Inline Roll: Refers to the rotation or angle of the geophone lines along the acquisition area in the inline direction, which could affect the coverage and sampling.

- Cross Line Roll: Refers to the rotation or angle of the geophone lines in the cross-line direction, similarly affecting the layout and coverage of the receivers in the cross-line direction.

- KEY CALCULATIONS

Multiplicity Calculation:

Multiplicity represents the number of times a specific subsurface point is sampled by unique combinations of source and receiver pairs. It is a critical measure of data redundancy and plays a key role in improving the signal-to-noise ratio in seismic data acquisition. The calculation of multiplicity involves the following steps:

- Identify Midpoints: For each source-receiver pair, calculate the midpoint of the ray path. The midpoint coordinates are given by:

- Bin Midpoints: Divide the survey area into a grid of bins. Each bin represents a specific subsurface region.

- Count Overlaps: For each bin, count the number of midpoint traces that fall within its boundaries. This count represents the multiplicity (fold) for that bin.

The resulting multiplicity values can be represented as a 2D matrix where each cell corresponds to a subsurface bin and contains its fold value. Areas with higher multiplicity indicate greater overlap of source-receiver pairs.

Offset Calculation:

Offset is the horizontal distance between a source and a receiver in the seismic survey. It is an important parameter because it affects the travel time of seismic waves, their angles of incidence, and the resolution of subsurface features. The calculation of offset involves:

- Source and Receiver Coordinates: For each source-receiver pair, identify their coordinates.

- Compute Distance: Use the Euclidean distance formula to calculate the offset for each pair:

- Offset Distribution: Organize the calculated offsets into bins or ranges for analysis. Near offsets (short distances) are useful for imaging shallow structures, while far offsets (long distances) are critical for deeper imaging.

Both multiplicity and offset values can be used to generate visualizations, such as color-coded maps or matrices, to provide insights into the survey design and data quality.

- DATA STRUCTURES

Multiplicity Data Structure: The multiplicity values are stored in a 2D array where each cell represents a specific bin in the survey grid. The value in each cell corresponds to the fold or number of overlaps for that bin. This data structure is indexed by the grid coordinates of the bins and allows for efficient access and visualization of the multiplicity distribution.

Offset Data Structure: The offset values are stored in a structure that maintains the calculated offset for each source-receiver pair. This structure is indexed by the source and receiver indices, enabling efficient retrieval and analysis of offset data across all source-receiver combinations.

- OUTPUTS

The system generates two sets of outputs to represent the calculated multiplicity and offset values:

- Multiplicity: A numerical summary of the multiplicity distribution across sources and geophones. Higher values indicate areas with more overlap in the acquisition geometry, helping to identify regions with denser coverage.

- Offset: A numerical representation of the offset (distance) between sources and geophones. This information is crucial for assessing the overall geometry of the seismic acquisition and ensuring appropriate coverage.

- MATHEMATICAL CONSIDERATIONS

The offset calculation assumes that both the sources and geophones are arranged on a regular 2D grid.

Multiplicity is modeled as a simplified function of the overlap between sources and geophones, using random integers to simulate variability in the number of times a geophone is triggered by a source.

While this approach simplifies some aspects of real-world seismic surveys (such as shot-receiver geometry), it serves as an illustrative model for understanding the basic relationships between source and receiver geometry.

- ALGORITHM STEPS

1.Initialize Variables:

- Create a 2D grid for the geophone receivers based on the geophone_distance (distance between adjacent geophones in meters) along the x and y axes.

- Define a source position at the origin (0,0) or based on your survey layout.

- Initialize parameters like geophone_line_distance (distance between geophone lines), number of channels per line, number of lines recorded at once, inline roll, and cross-line roll to define the survey geometry.

2.Calculate Offset:

- For each receiver point in the 2D grid, calculate the offset, which is the straight-line distance between the source and the receiver.

- The offset for a receiver located at (xreceiver,yreceiver) and the source at (xsource,ysource) is calculated using the Euclidean distance formula.

- Ensure the min_offset and max_offset constraints are respected, filtering the calculated offsets to be within the valid range.

3.Calculate Multiplicity:

- For each receiver point in the 2D grid, calculate multiplicity. This represents how many sources will overlap with that specific receiver.

- For each point on the grid, count the number of source-receiver pairs to determine the multiplicity.

- The multiplicity value can be adjusted based on inline roll and cross-line roll to account for rotated or skewed acquisition geometries.

4.Plot the Survey: Use a plotting function (e.g., matplotlib) to visualize the 2D grid, showing:

    - The positions of the geophones on the grid.

    - Offset values as a heatmap or color-mapped grid, showing the distance between the source and each receiver.

    - Multiplicity values as a heatmap or color-mapped grid, representing the number of times each geophone is used in the survey.

    - The effects of inline roll and crossline roll should be visualized as a rotation or shift in the grid layout.

- USER INTERFACE

The tool operates via a Python script. Users provide the following input parameters:

- Area

- Geophone Distance

- Geophone Line Distance

- Number of Channels per Line

- Number of Lines Recorded at One Time

- Inline Roll

- Cross Line Roll

The results are printed to the console, and heatmaps are shown using matplotlib for visual inspection.

- TESTING AND VALIDATION

The tool's outputs can be validated by comparing:

Multiplicity: Check if the randomness of multiplicity values produces expected patterns, where overlap increases towards the bottom-right corner of the grid.

Offset: Ensure that the offset values increase smoothly with distance from the origin, as per the Euclidean distance formula.

Test cases should involve varying the number of sources, geophones, and spacing parameters to ensure that the system works correctly across different configurations.

- DEPENDENCIES

numpy: For efficient array manipulation and calculations.

matplotlib: For visualizing the multiplicity and offset distributions as heatmaps.

- CONCLUSION

This tool provides a simple yet effective way to simulate the key parameters (multiplicity and offset) in a 3D seismic acquisition system. It is a valuable tool for understanding the basic principles of source-receiver geometry in seismic surveys. The visualizations produced by the tool can help survey planners assess the coverage and effectiveness of their seismic acquisition layout.